Another tunnel, the Papago Freeway Drainage Tunnel in Phoenix, used both compaction and chemical grouting during tunneling to control surface settlement, ground runs, and the development of large, open chimneys to the ground surface.

The project was constructed between 1984 and 1987.  It consisted of three soft-ground tunnels.  These tunnels were referred to as the north, east, and west tunnels.  The total length of the tunnels was approximately 10.5 km (6.5 m).  Of the three tunnels, only the west tunnel was grouted.  It was 4.3km (13,970 ft) long with an excavated diameter of 7.6m (25 ft) and a finished diameter of 6.4 m (21 ft).  The crown of the tunnel was between 10.1 and 13.7 m (33 and 45 ft) below the surface.  All three tunnels were excavated through coarse alluvial sand-gravel-cobble deposits in downtown Phoenix.  All but a third of the east and west tunnel lengths were driven above the groundwater table.  In the portion of tunnel excavation below the groundwater table, the groundwater levels were drawn down to below tunnel invert using several large-diameter, high-capacity wells.

The contractor selecte4d a shield machine with a digger arm.  Almost immediately after starting the east tunnel, which had no ground modification associated with the excavation, a series of large ground runs occurred.  The runs resulted in large surface settlements on the order of 2-3 m (6-10 ft) and in large, open chimneys to the ground surface.  Since the tunnel alignment ran under highway right-of-way with little surface development, no surface structural damage was sustained.  In an effort to control these runs, the contractor modified the tunnel machine by adding poling plates and breasting boards.  The east tunnel was completed with these modifications.

Based on the ground movements experienced during the east tunnel drive, and the fact that the west tunnel alignment passed under highly developed properties, a ground modification program was considered.  A risk assessment was performed based on the types of development along the west alignment.  The development included streets and pavements, high-voltage electric lines, high-pressure gas lines, and fiber-optic telecommunications cable, as well as residential and commercial buildings.  Based on the risk assessment and the experience gained on the east tunnel drive, a program of compaction and chemical grouting was developed for the west tunnel.

The compaction grouting was performed from the surface as the tunnel advanced.  Grout holes were drilled on 3 m (10 ft) centers along the centerline of the tunnel alignment to within approximately 3 m (10 ft) above the crown of the tunnel.  Each hole was fitted with a 75mm (3 in.) diameter grout pipe.  Compaction grouting was started on a hole just after the tail shield passed beyond a grout pipe.  At first, noncementitious grout was used consisting of well-graded silty sand, fly ash, and water.  The mixture had a slump of approximately 50 mm (2 in.).  The mix was later changed to a readily available native sandy silt without fly ash.  This mix was used for the remainder of normal tunneling operations.  However, when ground losses in excess of 76.5 m3 (100 cu yd) occurred at the face, one bag of portland cement was added to every 1.5m3 (2 cu yd) of grout mixed.  The cement was added to strengthen the ground and reduce the chances of grout flow into the heading around the face of the shield.  Lyman (1988) reported the grouting program was effective in limiting near-surface ground movement.  This effectively minimized the risk of damage to utilities, street pavement, and nearby buildings throughout most of the west tunnel alignment.

As with jet grouting, compaction grouting is a specialized type of grouting requiring special equipment and procedures.  It is therefore performed by a limited number of contracting firms.  These specialty contractors usually design and construct the compaction grouting system to meet the design engineer's or prime contractor's requirements.  As with jet grouting, to maximize use of this method specialty contracting firms need to be contacted early in the design process.  A performance specification, as opposed to a method specification, is advised when compaction grouting is specified.

| Title: | DESIGN AND CONSTRUCTION OF INTERSTATE 10 DRAINAGE TUNNELS IN PHOENIX |
| --- | --- |
| Accession Number: | 00476073 |
| Record Type: | Component |

| Abstract: | Drainage of the depressed Interstate 10 inner-loop extension in Phoenix, Arizona, required the construction of a conveyance system composed of pressure tunnels, 14 to 21 ft in diameter, beneath the city. The geologic and engineering parameters specific to this project and the method of construction used in stream-transported alluvial soils are described. Minimization of surface settlement and concern for lowering of the groundwater table, which is almost completely controlled by the nearby Salt River, influenced construction methods and contractual provisions. |
| --- | --- |
| Supplemental Notes: | This paper appeared in Transportation Research Record No. 1150, Tunneling. |
| TRIS Files: | HRIS |
| Pagination: | p. 37-42 |
| Authors: | Smirnoff, T P |
| Features: | Figures (9); Tables (3) |
| Monograph Info: | See related components |
| Corporate Authors: | - Transportation Research Board 500 Fifth Street, NW Washington, DC 20001 USA |
| Availability: | - Transportation Research Board Business Office 500 Fifth Street, NW Washington, DC 20001 USA Order URL: http://onlinepubs.trb.org/onlinepubs/out_of_print.htm - Find a library where document is available Order URL: http://worldcat.org/isbn/0309046556 |
| ISBN: | 0-309-04655-6 |
| Publication Date: | 1987 |
| Serial: | Transportation Research Record Issue Number: 1150 Publisher: Transportation Research Board ISSN: 0361-1981 |
| Index Terms: | Alluvium; Construction management; Diameter; Drainage structures; Engineering geology; Groundwater; Pressure tunnels; Water table; Drainage systems; Groundwater level |
| Subject Areas: | H25: STRUCTURES DESIGN AND PERFORMANCE H33: CONSTRUCTION |

LOCATION; Science-Engineering Library. Call #; TE7 .H5 no.1150. Title; Tunneling...

Other Author; National Research Council (U.S.). Transportation Research Board.. Series; Transportation research record, 0361-1981 ; 1150.. Publisher; Washington, D.C. : Transportation Research Board, National Research Council, 1987 [i.e. 1988]. Subjects; Tunneling.. Tunnels.. Note; Papers prepared for the 67th Annual Meeting of the Transportation Research Board.. Includes bibliographies.. Description; iv, 48 p. : ill. ; 28 cm.. ISBN; 0309046556..

|  | Title; DRAINAGE TUNNELS SAVE FREEWAY LINK. Accession No; 00458293. Authors; Miller, R D; Schwab, R E; Smirnoff, T P. Journal Title; Civil Engineering Vol. 55 No. 8. Corp. Authors / Publisher; American Society of Civil Engineers ; American Society of Civil Engineers. Publication Date; 19850800. Description; p. 72-74. Abstract; A depressed freeway extension through Phoenix, Arizona, threatened by flooding will be drained by a 6.5 mile tunnel network. Three pressure tunnels--two acting as long inverted siphons--will capture and divert the runoff. The east and west tunnels, 13,900 feet and 13,700 feet long, will run parallel and convey runoff collected from the northern half of the drainage area beneath the roadway directly to the river. A 6,700 foot tunnel located on the north side of the depressed freeway will intercept runoff north of I-10 and channel it into the west tunnel.. TRT Terms; Depressed highways ; Drainage structures ; Flood protection ; Runoff ; Surface drainage ; Tunnels. Identifier Terms; Interstate Highway System. Other Terms; Diversion tunnels; Flood control; Highway drainage. Subject Areas; H22 HYDROLOGY AND HYDRAULICS; I26 Water run-off - freeze-thaw. Availability; Linda Hall Library. Document Source; Engineering Index Source Data: EIX851200009. TRIS Files; HRIS. Database; TRIS Online. |
| --- | --- |

Civil Engineering—ASCE, Vol. 55, No. 10, October 1985, pg. 40

## Drainage Tunnels Save Freeway Link

by Robert D. Miller, Member, ASCE, (Partner, Howard Needles Tammen & Bergendoff, Phoenix, Ariz.), Richard E. Schwab, Member, ASCE, (Chief, Hydraulics Section, Howard Needles Tammen & Bergendoff, Kansas City, Mo.), and Timothy P. Smirnoff, Member, ASCE, (Manager, Tunnel and Underground Engineering Dept., Howard Needles Tammen & Bergendoff, Kansas City, Mo.)

Civil Engineering—ASCE, Vol. 55, No. 8, August 1985, pp. 72-74

A 15 mile section through downtown Phoenix will complete Interstate 10. After years of delay, the project is now close to completion. The project required construction of 6.5 miles of drainage tunnels. The tunnels were selected from some 21 alternate designs to solve the problem of drainage, particularly impacting a 3 mile depressed freeway section. Local rainfall conditions, a high degree of urbanization and the location of the freeway between an extensive portion of watershed and the natural outlet for runoff would create potential for flooding. Three tunnels will convey runoff without disrupting traffic and the downtown economy. The cost of this alternative is half of that of conventional drainage systems. Two of the tunnels with outfalls at the Salt River will act as inverted siphons, thereby overcoming a problem caused by the flat topographic relief of the Phoenix Basin. Runoff is intercepted above the freeway by a conventional near-surface sewer system and conveyed into the tunnels. Runoff falling on the depressed freeway will be pumped to the near-surface conveyance and on into the tunnels. Since the tunnels will be subjected to surge conditions, they are designed as pressure conduits.